*----*----*----*----*----*----*----*----*----*----*----*----*----*

Project Name    : hur01

Aircraft Number : N43RF

Flight Date     : 011003I

Processing Date : 30-Oct-2001

Data Processor  : barryd

Raw Data File   : /acdata/hur01/data_raw/011003I_s.dat

Processed Data  : /acdata/hur01/data_mat/011003Is_P.mat

Diary File      : /acdata/hur01/docs_proc/011003Is_P.doc

*----*----*----*----*----*----*----*----*----*----*----*----*----*

Processing Information

Reference Temperature Sensor: ttf1

Reference Humidity Sensor   : dpaocf

Winds are Calculated from   : Fuselage (INE2)

Calibration Formulas:

Angle of Attack : aaf=((pdaf./pqf1)+ (0.09) )/0.131;         JK 10/1

Sideslip Angle  : ssaf=((pdsf./pqf1c)+ (-0.008929) )/0.1786;       J

---------------------------------------------------------------------

N43RF AOC Empirical Adjustments  30-Oct-2001

Formula: X_adj= X_orig + Empirical Adjustment

Variable   Reference	Adjustment	Cal Flight

--------   ---------	----------	----------

Total	     ttf1	(0.00 C)	XXXXXX

Temperature

Dewpoint    dpaocf	(0.00 C)	XXXXXX

Temperature

Sea Surface  prt5   	(0.00 C)	XXXXXX

Temperature

Dynamic      pqf1  	(1.26 mbar)	XXXXXX

Pressure      pqr     	(0.79 mbar)	XXXXXX

Static			 See		XXXXXX

Pressure		Below

Static Pressure Correction from Trailing Cone Empirical Data:

Correction Formulas:  s_psfc = psf - ps_defect

ps_defect = A1*pqf1 + A2*pqf1^2 + A3*pqf1^2.5 + A4*pqf1^3

(all pressures in mb)

Coefficients:  	A1 =  0.17701

(from N43RF)   	A2 = -0.0129067

A3 =  0.00190547

A4 = -0.000078284

---------------------------------------------------------------------

-----------------------------------------------------

011003I - HP Time Period of Slow Tape Operation:

Starting Time --> 15:21:21 UTC

Ending Time   --> 23:20:30 UTC

Total time in seconds: 28750.0

Total time in minutes: 479.17

Total time in hours  : 7.99

OR, 7 hour(s), 59 minute(s), and 10 second(s)

-----------------------------------------------------

011003I - Contiguous Time Check on HP Time.

The slow tape is contiguous: 15:21:21 - 23:20:30 UTC