*----*----*----*----*----*----*----*----*----*----*----*----*----*

Project Name    : hur01

Aircraft Number : N42RF

Flight Date     : 011103H

Processing Date : 08-Nov-2001

Data Processor  : barryd

Raw Data File   : /acdata/hur01/data_raw/011103H_s.dat

Processed Data  : /acdata/hur01/data_mat/011103Hs_P.mat

Diary File      : /acdata/hur01/docs_proc/011103Hs_P.doc

*----*----*----*----*----*----*----*----*----*----*----*----*----*

Processing Information

Reference Temperature Sensor: ttf1

Reference Humidity Sensor   : dpbuck

Winds are Calculated from   : Fuselage (INE2)

Calibration Formulas:

Angle of Attack : aaf=((pdaf./pqf1)+ (0.34) )/0.161;         JK 9/24

Sideslip Angle  : ssaf=((pdsf./pqf1c)+ (0.0617) )/0.1764         JK

---------------------------------------------------------------------

N42RF AOC Empirical Adjustments  08-Nov-2001

Formula: X_adj= X_orig + Empirical Adjustment

Variable   Reference	Adjustment	Cal Flight

--------   ---------	----------	----------

Total	     ttf1	(0.00 C)	XXXXXX

Temperature

Dewpoint    dpbuck	(0.00 C)	XXXXXX

Temperature

Sea Surface  prt5   	(0.00 C)	XXXXXX

Temperature

Dynamic      pqf1  	(1.31 mbar)	XXXXXX

Pressure      pqr     	(0.87 mbar)	XXXXXX

Static			 See		XXXXXX

Pressure		Below

Static Pressure Correction from Trailing Cone Empirical Data:

Correction Formulas:  s_psfc = psf - ps_defect

ps_defect = A1*pqf1 + A2*pqf1^2 + A3*pqf1^2.5 + A4*pqf1^3

(all pressures in mb)

Coefficients:  	A1 =  0.15211

(from N42RF)   	A2 = -0.0122277

A3 =  0.00185083

A4 = -0.000077304

---------------------------------------------------------------------

-----------------------------------------------------

011103H - HP Time Period of Slow Tape Operation:

Starting Time --> 21:49:11 UTC

Ending Time   --> 07:23:00 UTC

Total time in seconds: 34430.0

Total time in minutes: 573.83

Total time in hours  : 9.56

OR, 9 hour(s), 33 minute(s), and 50 second(s)

-----------------------------------------------------

011103H - Contiguous Time Check on HP Time.

There are 2 contiguous time blocks on the slow tape.

Please confirm these numbers,

Record      1 - HP Time: 21:49:11 UTC

Record    840 - HP Time: 22:03:10 UTC

Record    841 - HP Time: 22:03:22 UTC

Record  34419 - HP Time: 07:23:00 UTC

HP Time Gap 1 : 12 Seconds