*----*----*----*----*----*----*----*----*----*----*----*----*----*

Processing Date:  08-Feb-2001

Data Processor :  barryd

Diary File     :  /acdata/pacjet/docs_proc/010112Is_P.doc

Total Number of Records: 11320

Reading Records:   500  1000  1500  2000  2500  3000  3500  4000  4500  5000  5500  6000  6500  7000  7500  8000  8500  9000  9500 10000 10500 11000 11500

==============================================================================

N43RF ADC Channel Assignments for 010112

ADC

Channel	 Variable	Instrument Description

00      lwcjw    	Liquid Water Content J-W          N43RF ABD 010116 Pacjet 2001

01      ttf1    	Rosemount Total Temp #1 FWD     SN 31416/amp #11 CAL DATE 083000

02      ttf2    	Rosemount Total Temp #2 AFT     SN a7665/amp #151 cal date 083000

03      dpgef    	General Eastern Dewpoint   SN 1201/ sn 52724/93602 cal date 083000

04      pdaf    	Attack Pressure Fuselage	  SN 877 CAL DATE 083000

05      pqaf    	Dynamic Attack Pressure Fuselage  SN 842 CAL DATE 083000

06      pdsf    	Sideslip Pressure Fuselage   	  SN 878 CAL DATE 083000

07      pqsf    	Dynamic Sideslip Pressure Fus     SN 861 CAL DATE 083000

08      psw    	Static Pressure Wingtip           SN 163 CAL DATE 083000

09      pqw    	Dynamic Pressure Wingtip          SN 163 CAL DATE 083000

10      trdprt5    	Surface PRT-5 Radiometer  L.Fus	  SN 588 CAL DATE 012000

11

12      tsrdco2    	Radiometric Air Temp (Side)       SN 642 CAL DATE 012000

13

14      azi2    	Vertical Acceleration - INE 2

15      azi1    	Vertical Acceleration - INE 1

16      pdar       	Attack Pressure Radome	          SN 2317 CAL DATE 011101

17      pdsr    	Sideslip Pressure Radome 	  SN 879 CAL DATE 011101

18      pqr    	Dynamic Pressure Radome  	  SN 286 CAL DATE 011101

19      ptr    	Total Pressure Radome 		  SN 260 CAL DATE 011101

20      ttr    	Total Temperature Radome          SN 3129/amp #125 cal date 012201

21

22

23

24      dpaocf    	ET Dewpoint Fuselage 	EdgeTech 137X  18267 w/1684 CAL DATE  083000

25

26      lyman    	AIR Lyman alpha

27      dpbuck    	BK Dewpoint Fuselage    Buck Research 1011C 313-P/92768 cal date 012201

28      dw3bal    	DW3 Balance

29

30

31      klwc    	King Liquid Water

32      psf    	Static Pressure	Fuselage	  SN 161 CAL DATE 083000

33      pqf1    	Dynamic Pressure #1 Fuselage 	  SN 161 CAL DATE 083000

34      pqf2    	Dynamic Pressure #2 Fuselage	  SN 862 CAL DATE 083000

35      tt1hc    	TT1 heater current

36      tt2hc    	TT2 heater current

37

38

39

40

41

42

43

44

45

46

47      pcab    	Cabin Pressure Rmnt 1201F   sn 190 cal date 083000

48      axbt1    	AXBT Channel 1

49      axbt2    	AXBT Channel 2

50      axbt3    	AXBT Channel 3

51

52

53

54

55

56

57

58

59

60

61

62

63      truprt5    	Up PRT-5 Radiometer           Barnes Up-Looking Radiometer 648 cal date 010401

64

65

66

67

68

69

70

71

72

73

74

75

76

77

78

79

==============================================================================

Time for reading, saving data:  1.03 minutes

No problems detected on yrhp or mohp or ddhp for 010112I_s

NO time DROPOUT (i.e., No dt<0) found in  010112I_s

NO REDUNDANT time (i.e., No dt==0) found in  010112I_s

DATA IN TAPE ARE CONTIGUOUS

BLOCKS 	      SAMPLES		       UTC

01	    00001-11320		15:42:41-18:51:20

View Figures of Raw Data? [yN]: n

********************************************************************************

Substituting dynamic pressure values < 35.0 mb at takeoff and landing with 35.0 mb.

********************************************************************************

Checking pqf1

-------------

Dynamic pressure values set to 35.0 mb for sample numbers below: 1999

Dynamic pressure values set to 35.0 mb for sample numbers above: 10173

***WARNING THIS DYNAMIC PRESSURE HAS VALUES THAT ARE TOO LOW (<15 mb)

OUTSIDE OF THE TAKEOFF AND LANDING PERIODS

IF THIS IS PART OF SPECIAL "STALL MANEUVERS" YOU MAY PROCEED,

ELSE, YOU SHOULD LOOK MORE CLOSELY AT THIS DYN. PRESSURE.

Type CTRL<C> to STOP the process or HIT ANY key to CONTINUE:

Checking pqf2

-------------

Dynamic pressure values set to 35.0 mb for sample numbers below: 1999

Dynamic pressure values set to 35.0 mb for sample numbers above: 10173

***WARNING THIS DYNAMIC PRESSURE HAS VALUES THAT ARE TOO LOW (<15 mb)

OUTSIDE OF THE TAKEOFF AND LANDING PERIODS

IF THIS IS PART OF SPECIAL "STALL MANEUVERS" YOU MAY PROCEED,

ELSE, YOU SHOULD LOOK MORE CLOSELY AT THIS DYN. PRESSURE.

Type CTRL<C> to STOP the process or HIT ANY key to CONTINUE:

Checking pqr

------------

Dynamic pressure values set to 35.0 mb for sample numbers below: 1999

Dynamic pressure values set to 35.0 mb for sample numbers above: 10168

***WARNING THIS DYNAMIC PRESSURE HAS VALUES THAT ARE TOO LOW (<15 mb)

OUTSIDE OF THE TAKEOFF AND LANDING PERIODS

IF THIS IS PART OF SPECIAL "STALL MANEUVERS" YOU MAY PROCEED,

ELSE, YOU SHOULD LOOK MORE CLOSELY AT THIS DYN. PRESSURE.

Type CTRL<C> to STOP the process or HIT ANY key to CONTINUE:

Checking pqsf

-------------

Dynamic pressure values set to 35.0 mb for sample numbers below: 1999

Dynamic pressure values set to 35.0 mb for sample numbers above: 10181

***WARNING THIS DYNAMIC PRESSURE HAS VALUES THAT ARE TOO LOW (<15 mb)

OUTSIDE OF THE TAKEOFF AND LANDING PERIODS

IF THIS IS PART OF SPECIAL "STALL MANEUVERS" YOU MAY PROCEED,

ELSE, YOU SHOULD LOOK MORE CLOSELY AT THIS DYN. PRESSURE.

Type CTRL<C> to STOP the process or HIT ANY key to CONTINUE:

Checking pqaf

-------------

Dynamic pressure values set to 35.0 mb for sample numbers below: 1999

Dynamic pressure values set to 35.0 mb for sample numbers above: 10177

***WARNING THIS DYNAMIC PRESSURE HAS VALUES THAT ARE TOO LOW (<15 mb)

OUTSIDE OF THE TAKEOFF AND LANDING PERIODS

IF THIS IS PART OF SPECIAL "STALL MANEUVERS" YOU MAY PROCEED,

ELSE, YOU SHOULD LOOK MORE CLOSELY AT THIS DYN. PRESSURE.

Type CTRL<C> to STOP the process or HIT ANY key to CONTINUE:

Checking pqw

------------

Dynamic pressure values set to 35.0 mb for sample numbers below: 1999

Dynamic pressure values set to 35.0 mb for sample numbers above: 10179

***WARNING THIS DYNAMIC PRESSURE HAS VALUES THAT ARE TOO LOW (<15 mb)

OUTSIDE OF THE TAKEOFF AND LANDING PERIODS

IF THIS IS PART OF SPECIAL "STALL MANEUVERS" YOU MAY PROCEED,

ELSE, YOU SHOULD LOOK MORE CLOSELY AT THIS DYN. PRESSURE.

Type CTRL<C> to STOP the process or HIT ANY key to CONTINUE:

The cutoff index numbers for pressures > 35.0 mb are: z_pqst:  1999

z_pqnd: 10168

dpegg   :	variable DOES NOT EXIST in workspace.

The quality control routines will NOT be ran on these variables:

axbt1   	axbt2   	axbt3

ddhp    	dpbuck  	dw3balV

hhhp    	hr159p  	klwcV

lwcjw   	lymanV  	mmhp

mohp    	pcab    	sshp

trdprt5V	truprt5 	truprt5V

tsrdco2 	tsrdco2V	tt1hcV

tt2hcV  		tt2hcV

------------------------------------------------------------------------

|		  DATA QUALITY CHECKS FOR 010112Is		       |

------------------------------------------------------------------------

| Name   |     < min      |     > Max      |  [d(var)/dt] >>   |  Pass |

------------------------------------------------------------------------

|pqr     | < 15 mb        |                | > 10 mb/s         |  NO   |

|pqw     | < 15 mb        |                | > 10 mb/s         |  NO   |

|pqf1    | < 15 mb        |                | > 10 mb/s         |  NO   |

|pqf2    | < 15 mb        |                | > 10 mb/s         |  NO   |

|pqaf    | < 15 mb        |                | > 10 mb/s         |  NO   |

|pqsf    | < 15 mb        |                | > 10 mb/s         |  NO   |

|pdar    |                |                |                   |  YES  |

|pdaf    |                |                |                   |  YES  |

|pdsr    |                |                |                   |  YES  |

|pdsf    |                |                |                   |  YES  |

|psf     |                |                | > 0.5 mb/s        |  NO   |

|psw     |                |                |                   |  YES  |

|ptr     | < 285 mb       |                | > 40 mb/s         |  NO   |

|hr159s  |                |                | > 50 m/s          |  NO   |

|hr232   |                |                |                   |  YES  |

|hi1     | < 0 m          |                |                   |  NO   |

|hi2     | < 0 m          |                |                   |  NO   |

|hg      | < 0 m          |                |                   |  NO   |

|hdgi1   |                |                |                   |  YES  |

|hdgi2   |                |                |                   |  YES  |

|pitchi1 |                |                |                   |  YES  |

|pitchi2 |                |                |                   |  YES  |

|rolli1  |                |                |                   |  YES  |

|rolli2  |                |                |                   |  YES  |

|dai1    | < -8 deg       | > 8 deg        | > 5 deg/s         |  NO   |

|dai2    | < -8 deg       | > 8 deg        | > 5 deg/s         |  NO   |

|azi1    |                |                |                   |  YES  |

|azi2    |                |                |                   |  YES  |

|lati1   |                |                |                   |  YES  |

|lati2   |                |                |                   |  YES  |

|latg    |                |                |                   |  YES  |

|longi1  |                |                |                   |  YES  |

|longi2  |                |                |                   |  YES  |

|longg   |                |                |                   |  YES  |

|vxi1    |                |                |                   |  YES  |

|vxi2    |                |                |                   |  YES  |

|vxg     |                |                |                   |  YES  |

|vyi1    |                |                |                   |  YES  |

|vyi2    |                |                |                   |  YES  |

|vyg     |                |                | > 8 m/s/s         |  NO   |

|vzi1    |                |                |                   |  YES  |

|vzi2    |                |                |                   |  YES  |

|vzg     |                |                |                   |  YES  |

|ttf1    |                |                |                   |  YES  |

|ttf2    |                |                |                   |  YES  |

|ttr     |                |                |                   |  YES  |

|dpegg   | variable       | does           | not               | exist |

|dpgef   |                |                |                   |  YES  |

|dpaocf  |                |                |                   |  YES  |

|trdprt5 |                |                |                   |  YES  |

------------------------------------------------------------------------

------------------------------------------------------------------------

| Name   |     < min      |     > Max      |  [d(var)/dt] >>   |  Pass |

------------------------------------------------------------------------

|pqr     | < 15 mb        |                | > 10 mb/s         |  NO   |

------------------------------------------------------------------------

Entering Routine to Correct:---->	pqr

If there is a REDUNDANT variable that could be

used as a reference or substitute ENTER it NOW

Please do one of the following:

- Enter EXACT NAME of Reference variable to be used

- Hit Return key for NO Reference variable

- Enter s to Skip Correction of pqr

- Enter q to quit this routine (qcheckall.m)

What is your choice:   q

Starting the NOAA P-3 Data Reduction Program -- preproc.m

Using MATLAB version 5.3.0.10183 (R11)

These are SLOW data only - to create a *s_P.mat file.

-----------------------------------------------------

Processing pacjet Data File:  010112I

Please Choose Processing Speed:

1. Low    - User Controlled Pauses

2. Medium - Ten Second Pauses

3. High   - No Pauses

Input 1-3 [3]: 3

-----------------------------------------------------

010112I - HP Time Period of Slow Tape Operation:

Starting Time --> 15:42:41 UTC

Ending Time   --> 18:51:20 UTC

Total time in seconds: 11320.0

Total time in minutes: 188.67

Total time in hours  : 3.14

OR, 3 hour(s), 8 minute(s), and 40 second(s)

-----------------------------------------------------

010112I - Contiguous Time Check on HP Time.

The slow tape is contiguous: 15:42:41 - 18:51:20 UTC

Contiguous Time Block Check -- Completed.

The 7 newly-calculated variables are:

a_timeSTART  s_timehp_ymdhms  s_timetp     z_min_size

a_timeEND    s_timehp_run     s_timetc_01

Choose a Reference Dewpoint Sensor:

1. dpaocf - AOC Dewpointer - Fuselage

2. dpbuck

3. dpgef  - General Eastern Dewpointer - Fuselage

Input 1-3 [1]:

Error in ==> /opt/uci.src/uci/mfiles/choose_refs.m

On line 131  ==>  dp = input(['\nInput 1-',num2str(no_dps),' [1]:  ']);

Error in ==> /opt/uci.src/uci/mfiles/aoc_offsets.m

On line 47  ==>   choose_refs

Error in ==> /opt/uci.src/uci/mfiles/a1_slowcals.m

On line 314  ==>    aoc_offsets

Error in ==> /opt/uci.src/uci/mfiles/preproc.m

On line 70  ==> 	a1_slowcals

Error in ==> /opt/uci.src/uci/mfiles/preproc_aoc.m

On line 28  ==> preproc

Error in ==> /opt/uci.src/uci/mfiles/aocproc.m

On line 124  ==> preproc_aoc	%Process data (GPS correction, winds, thermo. variables etc...)

>>

>>

>>

>>

>> aocproc

Please enter your name: barryd

Please enter project name (e.g.: vortex): pacjet

Please enter flight name (e.g.: 941113H): 980223H