NOAA G-IV

WINTER STORMS 2006

Flight 5 – Track 29

PANC – PANC

Flight ID: 060202N

Sensor or system						Number or Name

Inertial (for wind derivation)  				VEW_PITR/VNS_PITR

Temperature Probe						AT3

Dew Point Probe						DPLC

Static and Dynamic Pressure					PS2C/QC2C

Project Directory / Constants File				601 / 49cal051

Notes:

There were no time/data glitches during this flight.

Turbulence was encountered at 2239Z.  This is reflected in the aircraft data.

AT2 was not installed for these missions.  DPLC was chosen as the primary dewpoint sensor because DPRC was greater than the ambient temperature during ascent and descent.  It also read 5-10 degrees warmer than DPLC through most of the flight.  Because of the ascent data, the data from DPLC appears to be the more accurate of the 2 sensors.

During ascent and descent, dynamic and differential pressures diverged due to location and air flow around the aircraft.  All data, specifically pressure and wind data, should be used carefully during these times.

DAP1 was once again operational.  However, raw dynamic attack pressure right (DAP2) was still used for derivations.

All other instruments worked properly.

Takeoff			Landing

Aircraft Static Pressure (mb)		992.0 mb		997.9 mb

Station Altimeter			n/a			29.56”

Flight Director:		Contact Paul Flaherty

Phone #:			(813) 828-3310 ext.  3094