Flight ID: 070913I

Sensor or system						Number or Name

Inertial								ine1

Accelerometer							acci1

Temperature Probe						tt2X

Dew Point Probe							tdm2X

Altitude (for vertical wind)						alti1 (GPS aided inertial alt)

Static Pressure							psfX (Rosemount Fuselage)

Dynamic Pressure						pqf1X (Rosemount Fuselage)

Time Source							Micro 99

Dropsondes Deployed						Good: 9  Bad: 0  (7 sent to NHC/HRD)

Local Met Data:			Takeoff (1953 Z)			Landing (0410 Z)

Aircraft Static Pressure		1004.3 	 mb			1005.8	 mb

Tower Pressure			1005	 mb			1006	 mb

Top of Form 1

Notes

Total Temp 3 sensor not installed

9 dropsondes deployed. 9 good. Sent 7 of the dropsondes out the aircraft. Did not send drops 2 or 8 to HRD/NHC

Data Gaps: 003650Z-003654Z, 003810Z-003816Z, 003840Z-003845Z, 003850Z-003858Z, 023510Z-023521Z

Removed spikes in lati1 and loni1 data at the end of the data gaps at 003650Z-003654Z and 003850Z-003858Z

There were errors in the heading data on the South legs which were caused by a calculation error in the data processing box located on the aircraft. The following times were replaced with a constant of 180º due to data spikes on a south heading: 233128Z-233133Z, 233145Z-233150Z, 231943Z-231946Z, 234811Z-234825Z, 245054Z-245254Z, 010205Z-010208Z, 011151Z-01242Z, 011337Z-011342Z, 011455Z-011522Z, 012121Z-012148Z. The data was saved in the variable: hdgi1X

The HG9550 radar altimeter data has a consistent 1% positive bias above 1,500 feet that was discovered after flight. The correction formula is:

(((reported altitude -  1,500) * 0.988) +1,500) = corrected altitude

This formula should only be needed for this hurricane season’s flight data as the problem should be corrected automatically during next season’s flights.

Removed data spike in psf & pqf1 at 003859Z. Saved patched data in variables psfx & pqf1x respectively.

There were periods where the dewpoint temperature exceeded the ambient temperature resulting in RH values > 100%. This was likely due to heavy rain, a wet-bulb effect on the total temperature sensor, and/or an artificial warming of the dewpoint sensor as it tried to burn off excess moisture. Corrections were not made to the data.