Flight ID: 070921I

Sensor or system						Number or Name

INE								1

Accelerometer							N/A

Temperature Probe						1

Dew Point Probe						2 (Edgetech)

Altitude (for vertical wind)					RA159(INE1 blended GPS)

Static Pressure							Rosemount Fuselage psf

Dynamic Pressure						Rosemount Fuselage pqf1

Time Source							Micro 99

Local Met Data:		Takeoff (2159Z)		Landing (0237Z)

Aircraft Static Pressure		 1011.6 mb		1013.4 mb

Tower Pressure			1011.9 mb		1014.9 mb

Top of Form 1

Notes:

There were no errors noted in the selected sensors.  The flight was shortened when the NHC required no further storm positions and data.

There were data gaps noted:  220041-220051, 222421-222440, 224747-224755, 021711-021730.

There were times during heavy precipitation events (e.g. eye wall penetrations) when the dew point exceeded ambient temperature3 yielding a RH greater than 100%.  This is probably due to a wet bulb effect on the total temperature probe and/or the dew pointer over heating while trying to remove excess moisture.  In these instances, no corrections were attempted.

SPECIAL NOTE:  In the netCDF file, 070921I_RC.nc, vertical ground speed (dpj_wgs), vertical air speed (dpj_was), and vertical wind speed (dpj_wz) were computed using Dr. Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

Flight Director:			Tom Shepherd

Phone #:			(813) 828-3310 ext. 3053