[image: image1.png]



[image: image2.png]



NOAA P-3 N43RF 

ARCPAC
Flight ID: 080403I
Sensor or system





Number or Name
INE







1


Accelerometer






1

Temperature Probe





1

Dew Point Probe





1 (Buck)
Altitude (for vertical wind)




Pressure Altitude
Static Pressure






Rosemount Fuselage (PSF)
Dynamic Pressure





Rosemount Fuselage (PQF1)
Time Source






Micro 99
Notes:

This flight was in two parts, i.e., Boulder, CO to Great Falls, MT and Great Falls, MT to Fairbanks, AK
Start:  173101

(Flt #2)  204101
End:   193700


 023900
There was one data gap during flight #2; 203600 - 203610.
Due to hardware changes in some of the navigational equipment, a number of parameters have been

adjusted in the netCDF file. RA159 and ALTI1 both display the blended altitude solution of the GPS/RINU navigation system. RA232 and ALTI2 altitude both display the HG9550 Radar Altitude, adjusted to the same scale as the previous INE altitude.

No Collins GPS data exist for this flight.  The new Novatel DL-3V precision GPS now occupies the Collins slots in the netCDF file (altcln, latcln, loncln).
Total temperature probe #3 (TT3) is inoperative, i.e., no sensor is installed.
On the first leg of the ferry flight, dew pointer #2 (Edgetech) was substituted for dew pointer #1 (Buck) with a 0.0 offset; 173825 – 173840.  On the second leg of the ferry, dew pointer #1had an area that was removed and “patched” with a 0.09 weighting:  210713 – 210908.
SPECIAL NOTE!!!
Vertical ground and vertical air speeds were computed 

using Dave Jorgensen’s vertical wind algorithm should be used for vertical

wind analysis.
 



Takeoff

Landing

Aircraft Static Pressure 

826.0 mb     

987.9  mb

Corrected Tower Pressure
823.2 mb  (KBJC)
989.0  mb  (PAFA)
Flight Director:


Tom Shepherd   (813) 828-3310 ext. 3053
