1

N43RF ERROR SUMMARY

Ocean Winds 2011

Mission # 2

Flight ID: 20110113I1

Sensor or system						Number or Name

INE	(for wind derivation)				INE1

Accelerometer						ACC1

Temperature Probe				          TT1

Dew Point Probe						TDM2 (EDGETECH)

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						          ALTI1

Constants File                                                           n43_11owv1.adc

Project Directory						/acdata/2011/MET/20110113I1

Notes:

There was a data gap from 162401Z - 162417Z during this flight.

All instruments worked optimally during the flight.

There was 1 total dropsonde deployed, 1 was good, 0 failed, and 0 were questionable.

There was 1 total AXBTs deployed, 1 was good, 0 failed, and 0 were questionable.

Flight notes:  The crew noticed a light dusting of salt aerosol on the windscreen, thus the mission was aborted.

There was 1 missed approach due to bad weather at CYHZ.

Takeoff/Landing data:  Data during landing and takeoff are potentially suspect.  It is recommended that ground data not be used for scientific analysis.

SPECIAL NOTE!!! The variable names dpj_wgs, dpj_was, and dpj_wz in the netCDF file represent vertical ground, vertical air, and vertical wind speeds respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

Takeoff		Landing

1609Z                   1918Z

Aircraft Static Pressure                976.0 mb	  981.0 mb

Corrected Tower Pressure           976.8 mb                981.0mb

Flight Director:	A. Barry Damiano/ Ian Sears     (813) 828-3310 ext. 3073/3039