NOAA G-IV N49RF ERROR SUMMARY

1

WINTER STORMS 2011

14 Jan 2011 Modified TRACK71-Repo (05WSC)

NCEP (PWT) PANC RJTY

Flight ID: 20110114N1

Reference Sensor or System	   			Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PSF.2

Slip Angle							SA.1

Temperature Probe				                     TTM.3

True Airspeed						TASF.3

Constants File                                                           49cal102

Flight Directory						acdata/2011/MET/20110114

Local Met Data			Takeoff (0813z)		Landing (1536z)

Aircraft Static Pressure		  1017.5 mb			   997.6 mb

Tower Pressure (corrected)	  1018.0 mb			   999.6 mb

__________________________________________________________________

Notes:

The only data gaps were in AltRa.1 from the APN-232 radar altimeter:

8:29:02- 8:33:20z (multiple gaps during this interval)

8:35:07z

9:34:20z

9:35:44z

10:46:28z

10:46:48z

15:10:23z

The ALTPA.1 (NACA Pressure Altitude) runs about 60 meters lower than a tight clustering of Air Data Computer and ADC Baro Corrected sources.

Angle of Attack sources: AaADDU.1 (from the Air Data Computer) was consistently about 0.5 degrees higher than either AA.1 or AA.2, or AaADDU.2. Spike in AA.1 on takeoff roll from 8:12:38 - 8:12:42z. Similar AA.1 spike after landing during rollout on the runway at 15:36:15z.

The GDIFF check: mean of 2185 meters of drift for INE #1 (with respect to the Novatel)

2606 meters for INE#2. In creation of higher MET parameters, INE #1 was selected.

The Left Dew Point sensor (TD.1) again read far too high through the entire flight (well above ambient temperature). TD.2 did a reasonably good job but is also too high during the cruise portion of flight at or above 41,000 feet (sondes RH near 5% and aircraft RH around 25%).

Crossing the dateline, there was a spread in times when each position reference marked the crossing with Inertial #1 sensing it first followed by a cluster of Inertial #2, the two Honeywell GPS sources and the Novatel, then a lag until it was recorded by Inertial #3.

Vertical Winds were slightly further away from a zero mean during the cruise portion of flight than in other recent missions, showing a small low bias (with a UZW.1 mean of -0.14). As expected, UWZ.1 shows a significant high bias during descent prior to landing.

All other flight level instruments worked optimally during the flight.

- 6 drop points assigned by NCEP

- 8 AVAPS I dropsondes deployed

- Of those 8 drops, 6 were good and transmitted, the first two attempts at Pt 6 failed (12:33:41 had no telemetry and 12:37:42z was a fast fall). The third attempt at Pt 6 was good.

- Of the 6 transmitted sondes, 2 coded surface winds with the other four coding 12, 13, 15 and 18 meters for a LAST WIND remark.

Flight Directors: Richard Henning and Jessica Williams (813) 828-3310 ext. 3086