NOAA G-IV N49RF ERROR SUMMARY

1

WINTER STORMS 2011

16 Jan 2011 TRACK76 (06WSW)

NCEP (PWT) RJTY RJTY

Flight ID: 20110116N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PSF.2

Slip Angle							SA.1

Temperature Probe				                     TTM.3

True Airspeed						TASF.3

Constants File                                                           49cal102

Flight Directory						acdata/2011/MET/20110116N1

Local Met Data			Takeoff (0735z)		Landing (1521z)

Aircraft Static Pressure		  987.6 mb			   991.6 mb

Tower Pressure (corrected)	  987.3 mb			   990.6 mb

__________________________________________________________________

Notes:

The only data gaps were in AltRa.1 from the APN-232 radar altimeter:

07:55:34-07:56:10z (multiple gaps during this interval)

12:51:06-12:51:07z

14:55:16-14:55:22z (multiple gaps during this interval)

The ALTPA.1 (NACA Pressure Altitude) ran about 80 to 90 meters lower than a tight clustering of Air Data Computer and ADC Baro Corrected sources during cruise.

Angle of Attack sources: AaADDU.1 (from the Air Data Computer) was consistently about 0.5 degrees higher than either AA.1 or AA.2, or AaADDU.2. Spike in AA.1 (up to 111.07 degrees) during takeoff roll at 07:34:14z 23 seconds prior to liftoff. No AA.1 spike after landing.

GDIFF check: Mean of 1703 meters of drift for INE #1 (with respect to the Novatel)

1882 meters for INE#2. In creation of higher MET parameters, INE #1 was selected.

The Left Dew Point sensor (TD.1) again read far too high through the entire flight (generally higher than all temperature sources). TD.2 did a reasonably good job (remaining at least 5 to 10 degrees below the ambient temperature) but is also too high during the cruise portion of flight at or above 41,000 feet (where sondes suggest an RH near 5% and the dew points generated by TD.2 yield an RH around 25%).

Vertical Winds showed a low bias during the cruise portion of flight (with a mean of -0.23 m/s) which became a more significant high bias during descent prior to landing.

All other flight level instruments worked optimally during the flight.

- 18 drop points assigned by NCEP

- 19 AVAPS I dropsondes deployed

- Of those 19 drops, 18 were good enough to create a WMO message and were transmitted. However, three of the good drops were transmitted with no heights: Drop Pt 1 at 08:08:53z failed at 880.5 mb, Drop Pt 2 at 08:25:25z failed at 909.1 mb and Drop Pt 6 at 09:27:36z failed at 780.2 mb. The first attempt at Drop Pt 4 at 09:12:42z was a Fast Fall but the backup was good. The first attempt at Pt 18 had no winds (at 14:44:05z) but the backup was good.

- Of the 18 good sondes, 15 coded surface winds.

Flight Directors: Richard Henning and Jessica Williams (813) 828-3310 ext. 3086