1

N43RF ERROR SUMMARY

Ocean Winds 2011

Mission #3

Flight ID: 20110117I1

Sensor or system						Number or Name

INE	(for wind derivation)				INE1

Accelerometer						ACC1

Temperature Probe				          TT1X

Dew Point Probe						TDM2X (EDGETECH)

Static Pressure						PSFX

Dynamic Pressure						PQF1X

Vert. Wind						          ALTI1

Constants File                                                           n43_11owv1.adc

Project Directory						/acdata/2011/MET/20110117I1

Notes:

There were data gaps from 170111Z – 170117Z, 182741Z – 182753Z, and 203111Z - 203710Z during this flight.

As data resumed at 170118Z following the data gap ending at 1701117Z, PSF, TT1, TDM2, PQAF, and PQF1 spiked.  They were modified using direct substitution with the following equations.

PSFX = PSW +13.8

TT1X = TT2 – 0.9

TDM2X = TDM1 + 3

PQAFX = PQF2 + 2.6

PQF1 = PQF2 + 1.9

During the second half of the flight, PQAF collected erroneous data.  It was modified using direct substitution from 203712Z – 002400Z with the following equations.

From 203712Z – 222600Z and 000201Z – 002400Z:

PQAFX = PQF2 + 0.7

From 222601Z – 000200Z

PQAFX = PQF2 – 2.0

From 213210Z – 213412Z, PSF collected erroneous data.  PSF was modified using direct substitution with the following equation.

PSFX = PSW + 4.7

During high altitude ferries to and from CYHZ and the scientific operations area, TDM2 collected erroneous data.  The data were modified using direct substitution from 171052Z – 181812Z and 222625Z – 234909Z with the following equation.

TDM2X = TDM1

There were 2 total AXBTs deployed, 2 were good, 0 failed, and 0 were questionable.

There was 9 total dropsondes deployed, 8 were good, 1 failed (fastfall), and 0 were questionable.

Takeoff/Landing data:  Data during landing and takeoff are potentially suspect.  It is recommended that ground data not be used for scientific analysis.

SPECIAL NOTE!!! The variable names dpj_wgs, dpj_was, and dpj_wz in the netCDF file represent vertical ground, vertical air, and vertical wind speeds respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

Takeoff		Landing

1651Z                   0013Z

Aircraft Static Pressure               1004.2 mb	             1009.5 mb

Corrected Tower Pressure           1004.2 mb                1009.6 mb

Flight Director:	A. Barry Damiano/ Ian Sears     (813) 828-3310 ext. 3073/3039