NOAA G-IV N49RF ERROR SUMMARY

1

WINTER STORMS 2011

2 Feb 2011 Modified TRACK74 (13WSW)

NCEP (PWT) RJTY RJTY

Flight ID: 20110202N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PSF.2

Slip Angle							SA.1

Temperature Probe				                     TTM.3

True Airspeed						TASF.3

Constants File                                                           49cal102

Flight Directory						acdata/2011/MET/20110202N1

Local Met Data			Takeoff (0739z)		Landing (1528z)

Aircraft Static Pressure		  1003.6 mb			     1005.3 mb

Tower Pressure (corrected)	  1003.9 mb			     1004.6 mb

__________________________________________________________________

Notes:

The only data gaps were in AltRa.1 from the APN-232 radar altimeter:

7:54:28 – 7:55:44z (multiple gaps during this interval)

8:18:04z

9:59:32z

10:35:26z

13:53:05z

14:50:15z

14:52:07z

15:03:31-15:03:36z (multiple gaps during this interval)

The ALTPA.1 (NACA Pressure Altitude) runs about 40-45 meters lower than a tight clustering of Air Data Computer and ADC Baro Corrected sources during cruise at 41K and 90-100 meters lower at 45K.

Angle of Attack sources: AaADDU.1 (from the Air Data Computer) was consistently about 0.5 degrees higher than either AA.1 or AA.2, or AaADDU.2. Spike in AA.1 prior to takeoff at 7:39:02z. Similar AA.1 spike after landing at 15:28:54z. In creation of higher MET parameters, PDAlphaF.2 was selected (under Attack Pressure Probe Selection choices) to utilize AA.2.

GDIFF check: Mean of 1051 meters of drift for INE #1 (with respect to the Novatel)

1611 meters for INE#2. In creation of higher MET parameters, INE #1 selected.

The Left Dew Point sensor (TD.1) again read far too high through the entire flight. TD.2 did a reasonably good job but is also too high during the cruise portion of flight at or above 41,000 feet (where sondes suggest an RH near 5% and the dew points generated by TD.2 yield an RH around 25%).

Vertical Winds showed a small low bias during the cruise portion of flight (with a UWZ.1 mean of -0.16) but then, as expected, show a significant high bias during descent prior to landing.

All other flight level instruments worked optimally during the flight.

- 19 drop points assigned by NCEP

- 19 AVAPS II dropsondes deployed

- All 19 were good and all 19 coded surface winds.

Flight Directors: Richard Henning and Jessica Williams (813) 828-3310 ext. 3086