NOAA G-IV N49RF ERROR SUMMARY

1

WINTER STORMS 2011

15 Feb 2011 Modified TRACK73 (21WSW)

NCEP (PWT) RJTY RJTY

Flight ID: 20110215N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE2

Static Pressure						PSF.2

Slip Angle							SA.1

Temperature Probe				                     TTM.3

True Airspeed						TASF.3

Constants File                                                           49cal102

Flight Directory						acdata/2011/MET/20110215N1

Local Met Data			Takeoff (0729z)		Landing (1533z)

Aircraft Static Pressure		  1004.9 mb			     1011.2 mb

Tower Pressure (corrected)	  1004.3 mb			     1010.9 mb

__________________________________________________________________

Notes:

There were two intervals of data gaps in AltRa.1 from the APN-232 radar altimeter:

07:43:39-07:45:10z (multiple gaps during this interval)

15:09:14z

The ALTPA.1 (NACA Pressure Altitude) ran about 80 meters lower than a tight clustering of Air Data Computer and ADC Baro Corrected sources.

Angle of Attack sources: AaADDU.1 (from the Air Data Computer) was consistently about 0.6 degrees higher than either AA.1 or AA.2, or AaADDU.2. Spike in AA.1 at 7:28:37z during takeoff roll from -35 to +49 degrees. Similar AA.1 spike during landing rollout from -123 to +162 degrees. In creation of higher MET parameters, PDAlphaF.2 was selected (under Attack Pressure Probe Selection choices) to utilize AA.2.

GDIFF check: Mean of 1330 meters of drift for INE #2 (with respect to the Novatel). The plot of GDIFF for INE#1 was incomplete (missing most of the flight). In creation of higher MET parameters, INE #2 selected.

The Left Dew Point sensor (TD.1) again read far too high through the entire flight. TD.2 did a reasonably good job but is also too high during the cruise portion of flight at or above 41,000 feet (where sondes suggest an RH near 5% and the dew points generated by TD.2 yield an RH around 25%).

Vertical Winds showed a small low bias during the cruise portion of flight (with a UWZ.1 mean of -0.15).

All other flight level instruments worked optimally during the flight.

- 19 drop points assigned by NCEP

- 19 AVAPS II dropsondes deployed

- All 19 were good and all 19 coded surface winds. However, for Drop Pt 9 at 10:31:22z there was an Early Launch Detect requiring the need to manually rebuild a launch line in the D File and Drop Pt 12 at 11:41:02z displayed an abnormally fast fall rate below 400 mb.

Flight Director: Richard Henning (813) 828-3310 ext. 3086