NOAA G-IV N49RF ERROR SUMMARY

1

WINTER STORMS 2011

16 Feb 2011 Modified TRACK69 (23WSW)

NCEP (PWT) RJTY RJTY

Flight ID: 20110216N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PSF.2

Slip Angle							SA.1

Temperature Probe				                     TTM.3

True Airspeed						TASF.3

Constants File                                                           49cal102

Flight Directory						acdata/2011/MET/20110216N1

Local Met Data			Takeoff (0733z)		Landing (1539z)

Aircraft Static Pressure		  1008.7 mb			     1010.2 mb

Tower Pressure (corrected)	  1009.5 mb			     1009.8 mb

__________________________________________________________________

Notes:

The only data gaps were two intervals in AltRa.1 from the APN-232 radar altimeter:

07:46:57-07:47:30z (multiple gaps during this interval)

15:09:14z

The ALTPA.1 (NACA Pressure Altitude) ran about 60 meters lower than a tight clustering of Air Data Computer and ADC Baro Corrected sources.

Angle of Attack sources: AaADDU.1 (from the Air Data Computer) was consistently about 0.6 degrees higher than either AA.1 or AA.2, or AaADDU.2. Spike in AA.1 during takeoff roll at 7:32:31 from -118 to +23 degrees. Similar AA.1 spike during landing rollout at 15:39:15z from -234 to +56 degrees. In creation of higher MET parameters, PDAlphaF.2 was selected (under Attack Pressure Probe Selection choices) to utilize AA.2.

GDIFF check: Mean of 3031 meters of drift for INE #1 (with respect to the Novatel)

3739 meters for INE#2. In creation of higher MET parameters, INE #1 selected.

The Left Dew Point sensor (TD.1) again read far too high through the entire flight. TD.2 did a reasonably good job but is also too high during the cruise portion of flight at or above 41,000 feet (where sondes suggest an RH near 5% and the dew points generated by TD.2 yield an RH around 25%).

Vertical Winds showed a small low bias during the cruise portion of flight (with a mean UWZ.1 of -0.19).

All other flight level instruments worked optimally during the flight.

- 13 drop points assigned by NCEP

- 13 AVAPS II dropsondes deployed

- All 13 were good and all 13 coded surface winds.

Flight Director: Richard Henning (813) 828-3310 ext. 3086