Flight ID: 20110222N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3

Attack Angle		AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE 1

Static Pressure							PSF.2

Slip Angle		SA.1

Temperature Probe						TTM.3

True Air Speed		TASF.3

Constants File		49cal102

Flight Directory		acdata/MET/2011/20110222N1

Local Met Data:			Takeoff (0732Z)		Landing (1526Z)

Aircraft Static Pressure		1007.7 mb			1012.4 mb

Tower Pressure (corrected)		1007.7 mb			1012.1  mb

Notes:

Static Pressure 2, PS.2, was within 0.5 mb of station pressure on takeoff and landing.

AA.1 spikes erroneously ~30 seconds before takeoff and after landing, which caused a spike in WS.  Default was changed to AA.2 to remove spikes.

Both SA 1 and 2 spike 30 seconds before takeoff and after landing.  SA.1 was left as default.

The Radar Altimeter, AltRa.1, has several spikes and is noisy throughout flight.  The Novatel GPS Altitude, AltGPS.3 is default.

The Left Dewpointer, TD.1, slowly cools to TD.2 after takeoff then warms too much after level flight to ~ -45C, then cools back to TD.2 after initial descent.

19 dropsondes launched (AVAPS II):  18 good, 1 bad.

All 18 good drops successfully transmitted and ingested into the 22/12Z model.