NOAA G-IV N49RF ERROR SUMMARY

1

WINTER STORMS 2011

24 Feb 2011 Track 80 (33WSW)

NCEP (PWT) RJTYRJTY

Flight ID: 20110224N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE2

Static Pressure						PSF.2

Slip Angle							SA.2

Temperature Probe				                     TTM.1

True Airspeed						TASF.3

Constants File                                                           49cal102

Flight Directory						acdata/2011/MET/20110224N1

Local Met Data			Takeoff (0730z)		Landing (1511z)

Aircraft Static Pressure		  1005.8 mb			     1006.4 mb

Tower Pressure (corrected)	  1007.7 mb			     1008.7 mb

__________________________________________________________________

Notes:

The only data gaps were intervals in AltRa.1 from the APN-232 radar altimeter:

07:46:39-07:47:29z (multiple gaps during this interval during climbout)

In creation of higher MET parameters, PDAlphaF.2 was selected (under Attack Pressure Probe Selection choices) to utilize AA.2.

GDIFF check: Mean of 2510 meters of drift for INE #1 (with respect to the Novatel)

1776 meters for INE#2. In creation of higher MET parameters, INE #2 selected.

The Left Dew Point sensor (TD.1) again read far too high through the entire flight. TD.2 did a reasonably good job but is also too high during the cruise portion of flight at or above 41,000 feet (where sondes suggest an RH near 5% and the dew points generated by TD.2 yield an RH between 25 and 35 %).

TTM.3 displayed an oscillation of 0.4 to 0.5C (with the period of oscillation varying greatly with some segments showing as small as a 3 minute period and another showing a 15 minute period) during cruise above 41K. TTM.1 is the default.

Vertical Winds showed a high bias during the cruise portion of flight (with a mean UWZ.1 of +0.29 for the interval from 08:20-14:07z).

All other flight level instruments worked optimally during the flight.

- 13 drop points assigned by NCEP

- 16 AVAPS II dropsondes deployed, (0923 and 0924z at Drop Pt #2 were fast falls, second backup was good, 1252z Drop Pt #12 was Fast Fall, backup was good)

- 13 drops good and transmitted for ingestion into 24/1200z model runs

Flight Director: Jessica Williams (813) 828-3310 ext. 3140 and LCDR Nancy Ash