NOAA G-IV N49RF ERROR SUMMARY

1

WINTER STORMS 2011

5 Mar 2011 TRACK 34 (37WSC)

NCEP (PWT) PHNL PHNL

Flight ID: 20110305N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PSF.2

Slip Angle							SA.2

Temperature Probe				                     TTM.1

True Airspeed						TASF.3

Constants File                                                           49cal102

Flight Directory						acdata/2011/MET/20110305N1

Local Met Data			Takeoff (1944z)		Landing (0150z)

Aircraft Static Pressure		  1017.3 mb			     1016.8 mb

Tower Pressure (corrected)	  1018.1 mb			     1016.6 mb

__________________________________________________________________

Notes:

There was a glitch in the AAMPS data system that caused a data gap for all analog and inertial data beginning 9 seconds after 00z that lasted 45 seconds.

Other than the interval described above: The only other data gaps were two intervals in AltRa.1 from the APN-232 radar altimeter:

20:01:23-20:02:04z (multiple gaps during this interval during climbout)

01:27:14- 01:27:36z (multiple gaps during this interval during descent to landing)

In creation of higher MET parameters, PDAlphaF.2 was selected (under Attack Pressure Probe Selection choices) to utilize AA.2.

GDIFF check: Mean of 1426 meters of drift for INE #1 (with respect to the Novatel)

1644 meters for INE#2. In creation of higher MET parameters, INE #1 selected.

TTM.3 displayed an oscillation of around 0.6C to 0.7C (with a period of approx 6 minutes) during cruise above 41K. TTM.1 is the default.

The Left Dew Point sensor (TD.1) again read far too high through the entire flight. TD.2 did a reasonably good job but is also too high during the cruise portion of flight at or above 41,000 feet (where sondes suggest an RH near 5% and the dew points generated by TD.2 yield an RH around 20%). TdM.2 did spike well above TA.1 during descent to landing.

Vertical Winds showed a small low bias during the cruise portion of flight (with a mean UWZ.1 of -0.09 during a 4 hour interval between 20:43z and 00:50z).

All other flight level instruments worked optimally during the flight.

- 13 drop points assigned by NCEP

- 14 AVAPS II dropsondes deployed, 1 Fast Fall (2317z Drop Pt #8) Backup was good

- 13 drops good and transmitted for ingestion into 06/0000z model runs

Flight Director: Jessica Williams (813) 828-3310 ext. 3140 and LCDR Nancy Ash