NOAA G-IV N49RF ERROR SUMMARY

1

WINTER STORMS 2011

8 Mar 2011 TRACK N/A (38WSC)

NCEP (PWT) PHNL PANC

Flight ID: 20110308N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PSF.2

Slip Angle							SA.2

Temperature Probe				                     TTM.1

True Airspeed						TASF.3

Constants File                                                           49cal102

Flight Directory						acdata/2011/MET/20110308N1

Local Met Data			Takeoff (2007z)		Landing (0238z)

Aircraft Static Pressure		  1018.2 mb			     999.1 mb

Tower Pressure (corrected)	  1019.2 mb			     999.9 mb

__________________________________________________________________

Notes:

There was a glitch in the AAMPS data system that caused a data gap for all analog and inertial data beginning 9 seconds after 00z that lasted 45 seconds.

Other than the interval described above: The only other data gaps were intervals in AltRa.1 from the APN-232 radar altimeter:

20:23:55-20:24:21z (multiple gaps during this interval during climbout)

There were seven additional instances of one second dropouts over the next 3.5 hours.

In creation of higher MET parameters, PDAlphaF.2 was selected (under Attack Pressure Probe Selection choices) to utilize AA.2.

GDIFF check: Mean of 3935 meters of drift for INE #1 (with respect to the Novatel)

13809 meters for INE#2. This is a much larger INE drift than normal. In creation of higher MET parameters, INE #1 selected.

TTM.3 displayed an oscillation of around 0.6C (with a period of oscillation varying from approx 3.5 minutes early in the mission to over 20 minutes late in the flight) during cruise above 41K. TTM.1 is the default.

The Left Dew Point sensor (TD.1) again read far too high through the entire flight. TD.2 did a reasonably good job but is also too high during the cruise portion of flight at or above 41,000 feet (where sondes suggest an RH near 5% and the dew points generated by TD.2 yield an RH around 20-25%). TdM.2 did spike well above TA.1 briefly during takeoff.

Vertical Winds showed a small high bias during the cruise portion of flight (with a mean UWZ.1 of +0.10 during a 5+ hour interval between 21:23z and 01:39z).

All other flight level instruments worked optimally during the flight.

- 13 drop points assigned by NCEP

- 15 AVAPS II dropsondes deployed, (2302z Drop Pt #6 no winds…backup was good, 0043z Drop Pt #12 winds lost below 35K…backup was good)

- 13 drops good and transmitted for ingestion into 09/0000z model runs

Flight Director: Jessica Williams (813) 828-3310 ext. 3140 and LCDR Nancy Ash