Flight ID: 20110313N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3

Attack Angle		AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE 1

Static Pressure							PSF.2

Slip Angle		SA.2

Temperature Probe						TTM.1

True Air Speed		TASF.3

Constants File		49cal102

Flight Directory		acdata/MET/2011/20110313N1

Local Met Data:			Takeoff (1930Z)		Landing (0311Z)

Aircraft Static Pressure		996.8mb			1016.1mb

Tower Pressure (corrected)		996.2mb			1016.5mb

Notes:

A gap in all data is seen covering the period beginning 00:00:09 UTC and ending 00:00:57 UTC.

Even though WS.1 is the reference horizontal wind speed, it should be noted that WsI has an inverted spike (at 22:50:38 UTC) which looks to be due to acceleration in the vertical direction (turbulence).  This produces a 7.5m/s difference between the two.

TD1 was warmer than ambient temp throughout the whole flight.  TD2 performed better staying below the ambient temp and will be used as the reference sensor.

AA.1 spiked at takeoff and landing which is consistently seen in other flights.  The magnitude of the spike at take off was 75º and landing 150º.  Therefore AA.2 was chosen

TTM3 oscillation of  1º C.  Used TTM1 as default.

Due to an oscillations seen in INE 2 and INE 3 Inertial 1 should be used.

13 dropsondes launched (AVAPS II):  13 good, 0 bad.

All 13 good drops successfully transmitted to be ingested into the 14/00Z model.