Flight ID: 20110322N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3

Attack Angle		AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE 3

Static Pressure							PSF.2

Slip Angle		SA.2

Temperature Probe						TTM.1

True Air Speed		TASF.3

Constants File		49cal102

Flight Directory		acdata/MET/2011/20110322N1

Local Met Data:			Takeoff (1921Z)		Landing (0301Z)

Aircraft Static Pressure		1019.4 mb			997.8mb

Tower Pressure (corrected)		1020.2 mb			996.9mb

Notes:

INE.3 was chosen due to INE.1 and INE.2 not seeing a dip in altitude from 01:53UTC to 02:05UTC

TTM.1 was chosen due to an oscillation with amplitude of about 0.5º-1.0ºC

TD1 was warmer than ambient temp throughout the whole flight.  TD2 performed better staying below the ambient temp and will be used as the reference sensor.

AA.2 was used as default because of spikes in AA.1 during takeoff and landing.  Amplitude of the spikes were 35º at 19:20UTC and 150º at 03:01UTC

16 dropsondes launched (AVAPS II):  14 good, 2 bad.

All 14 good drops successfully transmitted to be ingested into the 23/00Z model.