Flight ID: 20110323N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3

Attack Angle		AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE 1

Static Pressure							PSF.2

Slip Angle		SA.1

Temperature Probe						TTM.1

True Air Speed		TASF.3

Constants File		49cal102

Flight Directory		acdata/MET/2011/20110323N1

Local Met Data:			Takeoff (2009Z)		Landing (0217Z)

Aircraft Static Pressure		1003.0 mb			1003.9 mb

Tower Pressure (corrected)		1002.3	mb			1002.6 mb

Notes:

AA1 has its typical spike during takeoff and landing.  AA2 is used as reference.

TD1 was 15-20ºC too warmer throughout time spent at flight level.  Makes it unrealistic so TD2 was chosen as reference dew point.

There is a 1.75mb difference in PtADDU1 and PtADDU2 at 23:07:30.

There is a gap in all data at 00:00:09 to 00:00:54.

14 dropsondes launched (AVAPS II):  14 good, 0 bad.

All 14 good drops successfully transmitted to be ingested into the 24/00Z model.