Flight ID: 20110325N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3

Attack Angle		AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE 1

Static Pressure							PSF.2

Slip Angle		SA.1

Temperature Probe						TTM.1

True Air Speed		TASF.1

Constants File		49cal102

Flight Directory		acdata/MET/2011/20110325N1

Local Met Data:			Takeoff (1948Z)		Landing (0304Z)

Aircraft Static Pressure		998.4mb			996.9 mb

Tower Pressure (corrected)		998.2mb			997.2mb

Notes:

There is a gap in all data from 00:00:08z until 00:00:54z

TTM.3 has a steady oscillation that can be seen when compared with TTM.1 and TTM. 4.  The magnitude of the oscillation is about 1.28ºC with a mostly constant period of about 12 min.  TTM.1 was chosen as the reference temperature.

TD1 was on average 8ºC warmer than the ambient temperature whereas TD2 was a more realistic 11ºC cooler at flight level ambient temperature therefore TD2 was chosen as reference.

18 dropsondes launched (AVAPS II):  18 good, 0 bad.

All 18 good drops successfully transmitted to be ingested into the 26/00Z model.