Flight ID: 20110326N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3

Attack Angle		AA.2

Dew Point Probe						TD.2

Dynamic Pressure						PQF.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE 1

Static Pressure							PSF.2

Slip Angle		SA.1

Temperature Probe						TTM.1

True Air Speed		TASF.1

Constants File		49cal102

Flight Directory		acdata/MET/2011/20110326N1

Local Met Data:			Takeoff (1950Z)		Landing (0252Z)

Aircraft Static Pressure		997.0 mb			996.6 mb

Tower Pressure (corrected)		996.8 mb			996.5 mb

Notes:

Data gap appearing from 00:00:08Z to 00:00:54Z

TTM.3 had an oscillation with a magnitude of about 0.25ºC and a period of about 20min.  TTM.1 was used as the reference due to the oscillation.

TASF.1 was chosen due to the TTM.3 oscillation (the oscillation shows up in TASF.3).

TDM.2 was the only dewpoint sensor reading realistic values.

18 dropsondes launched (AVAPS II):  18 good, 0 bad.

All 18 good drops successfully transmitted to be ingested into the 27/00Z model.