Flight ID: 20110327N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3

Attack Angle		AA.2

Dew Point Probe						TDM.2

Dynamic Pressure						PQF.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE 1

Static Pressure							PSF.2

Slip Angle		SA.1

Temperature Probe						TTM.1

True Air Speed		TASF.1

Constants File		49cal102

Flight Directory		acdata/MET/2011/20110327N1

Local Met Data:			Takeoff (1931Z)		Landing (0254Z)

Aircraft Static Pressure		996.5 mb			996.3 mb

Tower Pressure (corrected)		996.5 mb			996.2 mb

Notes:

INE3 was showing much larger amplitude oscillations as compared to INE1 and INE2.  For example when comparing LatGPS2 to LatI3 the oscillation amplitude started off at approx 0.2º whereas when comparing LatI1 or LatI2 the oscillation was on the order of 0.01º.

TTM3 had an oscillation throughout the flight therefore TTM1 was selected in its place as reference (this oscillation is also seen in TASF3 because TASF3 uses TT3 in calculations so TASF1 was selected as reference).

19 dropsondes launched (AVAPS II):  19 good, 0 bad.

All 19 good drops successfully transmitted to be ingested into the 28/00Z model.