Flight ID: 20110328N1

Sensor or system						Number or Name

Accelerometer							AccZI.2

Altitude 							AltGPS.3

Attack Angle		AA.2

Dew Point Probe						TDM.2

Dynamic Pressure						PQF.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE 2

Static Pressure							PSF.2

Slip Angle		SA.1

Temperature Probe						TTM.1

True Air Speed		TASF.1

Constants File		49cal102

Flight Directory		acdata/MET/2011/20110328N1

Local Met Data:			Takeoff (1932Z)		Landing (0241Z)

Aircraft Static Pressure		999.5 mb			996.5 mb

Tower Pressure (corrected)		999.2 mb			996.2 mb

Notes:

By comparing GDIFF1 and GDIFF2, GDIFF2 shows less inertial drift and it can also be seen when comparing other inertial parameters.  INE2 (including AccZI2) was chosen to be the default.

Early in the flight, when changing altitudes, ALTPA seems to take about 10 minutes to stabilize at that pressure level which can be seen when compared to AltPaADDU.  ALTPA still used as reference however.

TDM2 was chosen as default, TDM1 remains unrealistic.  TDM1 is about 10-15ºC warmer than TDM2 at flight level and about 15ºC warmer than TA1.

Oscillations are seen in the TTM3 data so TTM1 was chosen as reference. (and TASF1 because the TTM3 oscillation shows up in the TASF3 calculation).

Small sporadic gaps in data were found in multiple ADDU parameters.  Problems is currently unknown and does not affect key parameters in other calculated parameters but should be noted.

19 dropsondes launched (AVAPS II):  19 good, 0 bad.

All 19 good drops successfully transmitted to be ingested into the 29/00Z model.