NOAA G-IV N49RF ERROR SUMMARY

1

HURRICANE 2011

28 June 2011 TDR FAT WSR-88D Comparison

(PSM) KMCFKMCF

Flight ID: 20110628N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.1

Dew Point Probe						TD.1

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PS.2

Slip Angle							SA.2

Temperature Probe				                     TTM.1

True Airspeed						TASF.1

Constants File                                                           49cal111

Flight Directory						acdata/2011/MET/20110628N1

Local Met Data			Takeoff (1614z)		Landing (1819z)

Aircraft Static Pressure		  unavailable			     1018.0 mb

Tower Pressure (corrected)	  1018.8 mb			     1018.1 mb

__________________________________________________________________

Notes:

The AAMPS data system was not available until 17:10:56z (56 minutes after takeoff).

TTM3 displayed around 0.4C oscillation with a period of about 3.5 minutes. TTM.1 is the default.

Both TdM.1 and TdM.2 exhibited significant supersaturation during the convective encounters at cruise altitude with dew point exceeding ambient temp (TA.1) by several degrees.

In creation of higher MET parameters, PDAlphaF.1 was selected (under Attack Pressure Probe Selection choices) to utilize AA.1.

GDIFF check: Mean of 814 meters of drift for INE #1 (with respect to the Novatel) 1515 meters for INE#2 (which was intermittent with many gaps). In creation of higher MET parameters, INE #1 selected.

Using AA.1 by selecting PDAlphaF.1 in the calculation of higher MET parameters, Vertical Winds during the cruise portion of flight from 17:28z to 17:52z at 41K showed a mean UWZ of +0.14 m/s. The UWZ off the plane looking at the raw A.nc file showed a very poor mean of +0.61.

All other flight level instruments worked optimally during the flight.

Flight Directors: Richard Henning (813) 828-3310 ext. 3086 and Jessica Williams