NOAA G-IV N49RF ERROR SUMMARY

1

HURRICANE 2011

22 July 2011 TDR OSAT WSR-88D Miami

(PSM) KMCFKMCF

Flight ID: 20110722N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.1

Dew Point Probe						TD.1

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PS.2

Slip Angle							SA.2

Temperature Probe				                     TTM.1

True Airspeed						TASF.1

Constants File                                                           49cal111

Flight Directory						acdata/2011/MET/20110722N1

Local Met Data			Takeoff (1720z)		Landing (1938z)

Aircraft Static Pressure		  1016.6 mb			     1016.2 mb

Tower Pressure (corrected)	  1018.5 mb			     1017.5 mb

__________________________________________________________________

Notes:

The AAMPS data system worked well with no data interruptions.

PSF.1 and PSF.2 displayed a mean of 2.15 millibars of difference during level cruise at 28K.

TTM3 displayed around 0.6C oscillation during cruise at 28K with a much longer period than seen in other flights this Summer (ranging from 6 minutes to over 15 minutes). TTM.1 is the default.

Both TdM.1 and TdM.2 exhibited supersaturation during three convective encounters at cruise altitude with dew point exceeding ambient temp (TA.1) by a few degrees.

In creation of higher MET parameters, PDAlphaF.1 was selected (under Attack Pressure Probe Selection choices) to utilize AA.1.

GDIFF check: Mean of 742 meters of drift for INE #1 (with respect to the Novatel) 1442 meters for INE#2. In creation of higher MET parameters, INE #1 selected.

Using AA.1 by selecting PDAlphaF.1 in the calculation of higher MET parameters, Vertical Winds during the cruise portion of flight from 17:40z to 19:10z at 28K showed a mean UWZ of 0.00 with a max of +8.75 m/s during a convective updraft at 18:55z. When using AA.2 instead (by selecting PDAlphaF.2)…UWZ looked extremely poor for the cruise with a very large positive bias (a mean UWZ.1 of +0.79 on the processed AC.nc file and a max of +9.32 during that convective updraft). The UWZ showed a mean of +0.84 off the plane looking at the raw A.nc file with that max updraft recorded as +11.62 m/s.

All other flight level instruments worked optimally during the flight.

Flight Directors: Richard Henning (813) 828-3310 ext. 3086 and Jessica Williams