1

N43RF ERROR SUMMARY

EPAC Tropical Storm DORA MISSION

Flight ID: 20110723I1

Sensor or system						Number or Name

INE	(for wind derivation)				INE1

Accelerometer						ACCI1

Temperature Probe				                    TT1

Dew Point Probe						TDM2X

Static Pressure						PSF

Dynamic Pressure						PQF1

Vert. Wind						          ALTI1

Constants File                                                          /acdata/adc/43_11v3.adc

Project Directory				/acdata/2011/MET/20110723I1

Notes:

There were no data gaps.

Dewpoint sensor 2 (TDM2 [EdgeTech]) generated erroneous data between

185101Z – 185215Z, 223221Z – 223239Z, and 223333Z – 223353Z. The erroneous data were replaced with dewpoint sensor 1 (TDM1 [ Buck]) output via direct substitution: TDM2 = TDM1.

During the flight there were instances where dewpoint temperature values exceeded derived ambient temperature values resulting in humidity values above 100%. These situations occurred during heavy precipitation events.

All other instruments worked optimally during the flight.

There were twenty-three (23) GPS dropsondes deployed...22 good 1 bad.

There were twenty-one (21) AXBTs deployed… 20 good 1 bad.

Takeoff/Landing data:  Data during landing and takeoff are potentially suspect.  It is recommended that ground data not be used for scientific analysis.

SPECIAL NOTE!!!    The variable names dpj_wgs, dpj_was and dpj_wz in the netCDF file represent vertical ground speeds , vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

Takeoff(1659Z)	Landing(0056Z)

Aircraft Static Pressure                 1012.9mb               1012.3 mb

Corrected Tower Pressure             1013.0mb	              1012.3mb

Flight Director:	Ian Sears       (813) 828-3310 ext. 3039