NOAA G-IV N49RF ERROR SUMMARY

1

HURRICANE 2011

8 Aug 2011 TDR Intercomparison with N42

(PSM) KMCFKMCF

Flight ID: 20110808N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.1

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PS.2

Slip Angle							SA.2

Temperature Probe				                     TTM.1

True Airspeed						TASF.1

Constants File                                                           49cal111

Flight Directory						acdata/2011/MET/20110808N1

Local Met Data			Takeoff (1912z)		Landing (2102z)

Aircraft Static Pressure		  1011.8 mb			     1012.5 mb

Tower Pressure (corrected)	  1013.1 mb			     1013.1 mb

__________________________________________________________________

Notes:

Other than the outage listed above: There were two instances of brief data gaps in AltRa.1, the APN-232 radar altimeter, at the following times:

17:49:37-17:50:36z (multiple gaps during this interval)

00:56:10z

TTM.3 displayed an oscillation of around 0.4C (with a period of just over 3 minutes) during cruise above 41K. TTM.1 is the default.

In creation of higher MET parameters, PDAlphaF.2 was selected (under Attack Pressure Probe Selection choices) to utilize AA.2.

GDIFF check: Mean of 1545 meters of drift for INE #1 (with respect to the Novatel) 1885 meters for INE#2. In creation of higher MET parameters, INE #1 selected.

Vertical Winds were very close to zero for most of the cruise portion of flight (with a mean UWZ.1 of +0.04 from 18:33z to 22:20z). However, associated with the AAMPS crash, there were two periods of extreme spiking where vertical wind vector data is unusable:

22:47-22:59z

23:32-23:48z

All other flight level instruments worked optimally during the flight.

Flight Directors: Richard Henning (813) 828-3310 ext. 3086 and Jessica Williams