1

N42RF ERROR SUMMARY

HURRICANE IRENE

Flight ID: 20110823H1

Sensor or system						Number or Name

INE	(for wind derivation)				INE1

Accelerometer						ACCI1

Temperature Probe				                    TT1

Dew Point Probe						TDM2

Static Pressure						PSFX

Dynamic Pressure						PQF1

Vert. Wind						          ALTI1

Constants File                                                          /acdata/adc/42_11v5.adc

Project Directory				/acdata/2011/MET/20110823H1

Special note: There was a major problem with the attack angle output during this mission that results in erroneous vertical winds for the entire mission.

Notes:

There were data gaps from 224801Z – 224805Z and 011011Z – 011012Z.

During the flight there were instances where dewpoint temperature values exceeded derived ambient temperature values resulting in humidity values above 100%. These situations occurred during heavy precipitation events.

Measured fuselage static pressures, PSF, generated erroneous data between

202745Z – 202746Z.  The erroneous data was substituted with values from the wingtip static pressure (PSW) .  The substitution equation follows.

PSFX =  PSW + 5.9

All other instruments worked optimally during the flight.

There were twenty-two (22) GPS dropsondes deployed.  19 were good 3 were bad.

Takeoff/Landing data:  Data during landing and takeoff are potentially suspect.  It is recommended that ground data not be used for scientific analysis.

SPECIAL NOTE!!! The variable names dpj_wgs, dpj_was, and dpj_wz in the netCDF file represent vertical ground, vertical air, and vertical wind speeds respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

Takeoff(1952Z)	Landing(0347Z)

Aircraft Static Pressure                 1010.6 mb               1013.1 mb

Corrected Tower Pressure             1011.1 mb	              1013.4 mb

Flight Director:	Ian Sears       (813) 828-3310 ext. 3039