NOAA G-IV N49RF ERROR SUMMARY

1

HURRICANE 2011

25 Aug 2011 Irene (NOAA9 2209A IRENE)

NHC (PHS) KMCFKMCF

Flight ID: 20110825N2

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.1

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PS.2

Slip Angle							SA.2

Temperature Probe				                     TTM.1

True Airspeed						TASF.1

Constants File                                                           49cal111

Flight Directory						acdata/2011/MET/20110825N2

Local Met Data			Takeoff (1718z)		Landing (0133z)

Aircraft Static Pressure		  1008.7 mb		               1007.0 mb

Tower Pressure (corrected)	  1009.7 mb			     1008.3 mb

__________________________________________________________________

Notes:

There was a glitch in the AAMPS data system that caused a data gap for all analog and inertial data beginning 7 seconds after 00z that lasted 45 seconds.

There were intervals of brief data gaps in AltRa.1, the APN-232 radar altimeter, at the following times:

17:44:30-17:45:11z (multiple gaps during this interval)

19:00:31z

01:26:23z

TTM.3 displayed an oscillation of around 0.5C (with a period of approx 3 minutes) during cruise above 41K. TTM.1 is the default.

In creation of higher MET parameters, PDAlphaF.2 was selected (under Attack Pressure Probe Selection choices) to utilize AA.2.

GDIFF check: Mean of 2579 meters of drift for INE #1 (with respect to the Novatel) 3482 meters for INE#2. In creation of higher MET parameters, INE #1 selected.

Vertical Winds were very close to zero during the cruise portion of flight (with a mean UWZ.1 of -0.01 from about 1930z to the AAMPS data crash at 21:27:31z). After the crash, good vertical wind data was not re-established until approximately 21:46z.

All other flight level instruments worked optimally during the flight.

- 36 drop points assigned by NHC for ingestion into 26/0000z models

- 38 AVAPS II dropsondes deployed, all 36 assigned points covered with an ob

- Drop #23 at 2239z died at 27,000 ft but backup was good. Drop #32 at 0022z was a Fast Fall but backup was good

- 34 of the 36 transmitted sondes coded surface winds.

Flight Directors: Richard Henning (813) 828-3310 ext. 3086 and LTJG Nick Morgan.