1

N43RF ERROR SUMMARY

HURRICANE IRENE

LANDFALL IN NORTH CAROLINA

Flight ID: 20110827I1         HRD MISSION

Sensor or system						Number or Name

INE	(for wind derivation)			                     INE1

Accelerometer				     	                     ACC1

Temperature Probe						TT1

Dew Point Probe						          TDM2 (EgdeTech)

Altitude (for vertical wind)					ALTI1 (GPS)

Static Pressure						           PSF

Dynamic Pressure						           PQF2

Constants File					         /acdata/adc/n43_hur11v3.adc

Project Directory                                                /acdata/2011/MET/20110826I1

Notes:

There were no data gaps.

Dewpoint sensor #2 (TDM2...EdgeTech) erroneous values were replaced with dewpoint sensor #1 (TDM1...Buck) values during the time frames

081820Z – 081828Z and 101024Z – 101028Z.

Dewpoint sensor #2 (TDM2...EdgeTech) erroneous values were removed and replaced using statistical techniques during the time frames 093200Z – 093400Z, 141300Z – 141500Z and 150600Z – 150800Z. Dewpoint sensor #3 values were used as a reference.

During the flight there were numerous instances where dewpoint temperature values exceeded derived ambient temperature values resulting in humidity values above 100%. These situations occurred during heavy precipitation events.

All other instruments worked optimally during the flight.

Twenty-four (24) dropsondes were deployed. Nineteen (19) were good and 5 were bad.

SPECIAL NOTE!!!    The variable names dpj_wgs, dpj_was and dpj_wz in the netCDF file represent vertical ground speeds , vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

Takeoff (0804Z)     Landing (1515Z)

Aircraft Static Pressure                  1007.4mb     	         1009.8mb

Corrected Tower Pressure              1007.6mb	         1009.7mb

Flight Director:            A. Barry Damiano   (813) 828-3310 ext. 3073