Flight ID: 20110902N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3

Attack Angle		AA.2

Dew Point Probe						TdM.2X

Dynamic Pressure						PQF.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE 2

Static Pressure							PSF.2

Slip Angle		SA.1

Temperature Probe						TTM.4

True Air Speed		TASF.4

Constants File		49cal111.doc

Flight Directory		acdata/MET/2011/20110902N1

Local Met Data:			Takeoff (1709Z)		Landing (0112Z)

Aircraft Static Pressure		1014.7 mb			1013.0 mb

Tower Pressure (corrected)		1014.7 mb			1014.1 mb

Notes:

TTM.3 had an oscillation throughout the flight with the same period. The amplitude of the oscillation decreased as altitude increased resulting in about 0.8°C amplitude from 40-45kft.  TTM.1 was ~ 5°C too warm on taxi and shortly after t/o until 1707Z and spiked from 193141Z – 193203Z.  TTM.4 performed best with no spikes and was selected as default

PQAlphaF.1 started climbing erroneously after heavy precipitation and was out from 1933Z – 0050Z, on descent.  PQAlphaF.2 (AA.2) was selected as default.

Dew point values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.  TdM.2 performed better during the first half of the flight with slightly colder temperature and lower amplitude spikes through cloud cover, but TdM.1 was cooler for the last few hours.  On descent TdM.1 spiked with erroneous values after 005619Z until block in.  Because of this TdM.2 was selected as default with a direct substitution TdM.2 = TdM.1 from 224942Z – 005527Z, the period where TdM.1 performed better.

All other instruments worked properly.

40 sondes launched (AVAPS II): 38 good, 2 bad, 38 sent and ingested into the 03/00Z model.