NOAA G-IV N49RF ERROR SUMMARY

1

HURRICANE 2011

14 Sept 2011 Global Hawk Sonde Intercomparison

(PHX) KMCFKMCF

Flight ID: 20110914N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.1

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PS.2

Slip Angle							SA.2

Temperature Probe				                     TTM.1

True Airspeed						TASF.1

Constants File                                                           49cal111

Flight Directory						acdata/2011/MET/20110914N1

Local Met Data			Takeoff (1027z)		Landing (1549z)

Aircraft Static Pressure		  1013.4 mb			     1013.6 mb

Tower Pressure (corrected)	  1013.7 mb			     1015.0 mb

__________________________________________________________________

Notes:

The AAMPS data system worked well with no data interruptions.

TTM.3 displayed an oscillation around 0.4C (with a period of 2-3 minutes) during cruise at 43K. TTM.1 is the default.

In creation of higher MET parameters, PDAlphaF.2 was selected (under Attack Pressure Probe Selection choices) to utilize AA.2.

GDIFF check: Mean of 2035 meters of drift for INE #1 (with respect to the Novatel) 2853 meters for INE#2. In creation of higher MET parameters, INE #1 selected.

Vertical Winds were close to zero for the cruise portion of flight (with a mean UWZ.1 of -0.06 from 11:06z to 14:58z).

All other flight level instruments worked optimally during the flight.

- 33 drop points assigned for intercomparison with Global Hawk

- All 33 drops were good and measured surface winds

- Drop #4 at 1126z had an early launch detect but the D file was successfully repaired

- None of the drops were transmitted but all 33 D files were provided to HRD/NASA

Flight Directors: Richard Henning (813) 828-3310 ext. 3086 and Nick Morgan