NOAA G-IV N49RF ERROR SUMMARY

1

HURRICANE 2011

26 Sept 2011 TDR OSAT near Miami

(PSM) KMCFKMCF

Flight ID: 20110926N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.2

Dew Point Probe						TD.1

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PS.2

Slip Angle							SA.2

Temperature Probe				                     TTM.1

True Airspeed						TASF.1

Constants File                                                           49cal111

Flight Directory						acdata/2011/MET/20110926N1

Local Met Data			Takeoff (1507z)		Landing (1655z)

Aircraft Static Pressure		  1012.2 mb			     unavailable

Tower Pressure (corrected)	  1014.4 mb			     1014.1 mb

__________________________________________________________________

Notes:

The AAMPS data system crashed on the way back to KMCF after the TDR intercomparison with the Miami WSR-88D was complete. No data after 16:29:26z.

TTM.3 displayed an oscillation ranging from around 0.4C to as large as 0.7C (with a period of 3-4 minutes) during cruise at 41K. TTM.1 is the default.

In creation of higher MET parameters, PDAlphaF.2 was selected (under Attack Pressure Probe Selection choices) to utilize AA.2.

GDIFF check: Mean of 3294 meters of drift for INE #1 (with respect to the Novatel) 3600 meters for INE#2. In creation of higher MET parameters, INE #1 selected.

Vertical Winds were very close to zero for most of the cruise portion of flight (with a mean UWZ.1 of -0.05 from 15:16z to 16:15z).

All other flight level instruments worked optimally during the flight.

Flight Directors: Richard Henning (813) 828-3310 ext. 3086 and Ian Sears