Flight ID: 20111026H1

Sensor or system						Variable Name

Inertial	(for wind derivation)					ine1

Accelerometer							acci1

Temperature Probe						tt1

Dew Point Probe						tdm2X

Altitude (for vertical wind)					alti1

Static Pressure							psf

Dynamic Pressure						pqf1

Constants file							/acdata/adc/2011/42_11v5.adc

Flight Directory						/acdata/2011/MET/20111026H1

Local Met Data:			Takeoff (1156Z)		Landing (1833Z)

Aircraft Static Pressure		 1017.8 mb			1017.4  mb

Tower Pressure			 1017.5 mb			1017.1  mb

Notes

Flight delayed 4 hours due to #3EDC failure.

Dewpointer #2 performed best overall, however it displayed erroneous data from 140708Z – 141150Z, during which time period tdm3 was directly substituted and 173531Z – 182239Z, where tdm1 was directly substituted. Tdm2X is default.

There were periods where the dewpoint temperature exceeded the ambient temperature in the storm environment, resulting in RH values > 100%. This was likely due to heavy rain and/or an artificial warming of the dewpoint sensor as it tried to burn off excess moisture. Corrections were not made to the data.

TT2, total temp #2 spiked downward erroneously from 131909Z – 132401Z.  TT1 is default.

Psf and psw were within 0.5mb of station pressure on takeoff and landing.

All other instruments appeared to work optimally.

17 Dropsondes launched

10 AXBTS launched

2 pennies