Flight ID: 20111026N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3

Attack Angle		AA.2

Dew Point Probe						TD.1

Dynamic Pressure						PQF.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE 1

Static Pressure							PSF.2

Slip Angle		SA.2

Temperature Probe						TTM.1

True Air Speed		TASF.1

Constants File		49cal102

Flight Directory		acdata/MET/2011/20111026N1

Local Met Data:			Takeoff (1723Z)		Landing (0151Z)

Aircraft Static Pressure		1015.9 mb			1014.5mb

Tower Pressure (corrected)		1017.1mb			1015.1 mb

Notes:

Data Gaps

-The radar altimeter (APN-232) has data gaps at 17:48z, 01:33z, and 0136z.  These occur during ascent to flight altitude and descent to landing.

-GPS1 has data gaps that begin near 21:49z and are on and off to 00:53z.

AA has erroneous spikes at the moment of take-off and landing.

Vertical winds (UWZ) at cruise altitude average -0.01m/s

It should be noted that it appears that PDAlpha1 and PDAlpha2 were each affected by some turbulence significantly different at 3 areas of the flight at 18:43z, 18:57z, and 22:03z.  This caused some difference in the agreements between AA1 and AA2.  AA2 will still be used as reference.

In comparison of GDIFF1 and GDIFF2, Inertial 1 appears to drift less over time than 2.  Therefore INE1 will be used in calculations.

28 dropsondes launched (AVAPS II):  26 good, 2 bad.

All 28 good drops successfully transmitted to be ingested into the 27/00Z model.