Flight ID: 20111111I1

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.1

Dew Point Probe						TdM.2X

Static Pressure							PSF.1

Dynamic Pressure						PQF.1

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2011/20111111I1

Constants File	              2011111111/AAMPSConfig/core/n43.xml

Local Met Data:			Takeoff (0703Z)		Landing (1034Z)

Aircraft Static Pressure		1011.3 mb			1008.5 mb

Tower Pressure (corrected)		1011.5 mb			1008.4 mb

Notes:

There were no data gaps.

Dewpoint sensor 2 (TdM.2 [EdgeTech]) generated erroneous data between

072747Z – 072807Z, and 095606Z – 095627Z. The erroneous data were replaced with dewpoint sensor 1 (TdM.1 [ Buck]) output via direct substitution: TdM.2x = TdM.1.

Takeoff/Landing data:  Data during landing and takeoff are potentially suspect.  It is recommended that ground data not be used for scientific analysis.

SPECIAL NOTE!!!    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds , vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 5 GPS dropsondes, 5 AXBT’s and 2 CTD’s deployed from the aircraft internally.