Flight ID: 20111113I1

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.1

Dew Point Probe						TdM.2X

Static Pressure							PSF.1

Dynamic Pressure						PQF.1

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2011/20111113I1

Constants File	              2011111131/AAMPSConfig/core/n43.xml

Local Met Data:			Takeoff (0312Z)		Landing (1233Z)

Aircraft Static Pressure		1011.7 mb			1008.2 mb

Tower Pressure (corrected)		1010.7 mb			1007.7 mb

Notes:

During the flight, the data system malfunctioned several times resulting in multiple 1 Hz data files.  The only data files that are useable are 20111113I1_C.nc and 20111113I1_H.nc.  These two files will remain separated for the time being, but will be discussed as a singular data set for the purposes of this error summary.  20111113I1_C.nc  includes the times 015526Z - 062410Z.  20111113I1_H.nc  includes the times 070658-123853Z.  Note the time gap between the two files.

Dewpoint sensor 2 (TdM.2 [EdgeTech]) generated erroneous data between

054017Z – 054028Z, 054837Z – 054908Z, 055220Z – 055227Z, 055838Z – 055802Z, 061538Z – 061602Z, 094424Z – 094429Z, 094516Z – 094521Z, 102007Z – 102030Z, and 121314Z – 121329Z. The erroneous data were replaced with dewpoint sensor 1 (TdM.1 [ Buck]) output via direct substitution: TdM.2x = TdM.1.

Takeoff/Landing data:  Data during landing and takeoff are potentially suspect.  It is recommended that ground data not be used for scientific analysis.

SPECIAL NOTE!!!    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds , vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 49 GPS dropsondes, 51 AXBT’s and 4 CTD’s deployed from the aircraft internally.