Flight ID: 20111122I1

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.1

Dew Point Probe						TdM.2

Static Pressure							PSF.1

Dynamic Pressure						PQF.1

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2011/20111116I1

Constants File	              20111122I1/AAMPSConfig/core/n43.xml

Local Met Data:			Takeoff (0206Z)		Landing (1208Z)

Aircraft Static Pressure		1009.6 mb			1007.0 mb

Tower Pressure (corrected)		1009.2 mb			1006.0 mb

Notes:

There were no data gaps.

PQR.1, dynamic pressure from the radome, drops out on descent from 041542Z – 042624Z.  PQF.1, fuselage dynamic pressure, is the default.

Dewpoint sensor 2, TdM.2 (EdgeTech) performed best, however, other dew-point sources performed poorly, providing little redundancy for comparison.  Dewpoint values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.

Takeoff/Landing data:  Data during landing and takeoff are potentially suspect.  It is recommended that ground data not be used for scientific analysis.

SPECIAL NOTE!!!    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds , vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 51 GPS dropsondes, 21 AXBT’s and 12 CTD’s deployed from the aircraft internally.