Flight ID: 20111124I1

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.1

Dew Point Probe						TdM.2

Static Pressure							PSF.1

Dynamic Pressure						PQF.1

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2011/20111124I1

Constants File	             	AAMPSConfig/core/n43.xml

Local Met Data:			Takeoff (0407Z)		Landing (1303Z)

Aircraft Static Pressure		1012.2 mb			1009.3 mb

Tower Pressure (corrected)		1011.2 mb			1008.5 mb

Notes:

There were no data gaps.

From 025000Z – 032600Z and again from 084900Z – 104600Z at altitudes above the freezing level in precipitation, the following radome measurements were erroneous likely due to ice on the radome:  PQR.1, PDBetaR.1, PDAlphaR.1 and PTR.1.  Default measurements are from the fuselage instruments so no fixes were made.

Dewpoint sensor 2, TdM.2 (EdgeTech) performed very well and did not exhibit any erroneous spikes.  However, dewpoint values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.

Total Temp 2, TTM.2, spiked down from 032243Z – 032323Z.  TTM.1 is default.

SPECIAL NOTE:    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 58 GPS dropsondes,  9 AXBT’s and 2 CTD’s deployed from the aircraft internally.