Flight ID: 20111130I1

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.1

Dew Point Probe						TdM.2

Static Pressure							PSF.1

Dynamic Pressure						PQF.1

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2011/20111130I1

Constants File	             	AAMPSConfig/core/n43.xml

Local Met Data:			Takeoff (0136Z)		Landing (1133Z)

Aircraft Static Pressure		1007.9 mb			1005.6 mb

Tower Pressure (corrected)		1007.2 mb			1005.4 mb

Notes:

There are two 1hz files for this flight due to a restart of the data system.  From 082748Z – 084439Z all data is suspect surrounding the data system restart.  There is no data from 082748Z – 082810Z and again during the restart from 083105Z – 083715Z.

The following radome measurements were erroneous likely due to ice on the radome (at altitudes above the freezing level in precipitation) from 042406Z – 043652Z :  PQR.1, PDBetaR.1, PDAlphaR.1 and PTR.1.  Default measurements are from the fuselage instruments so no fixes were made.

Dew point sensor 2, TdM.2 (EdgeTech) spiked during descent into landing from 112055Z – 112721Z.  Also, dew point values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.

The Novatel GPS position data (AltGPS.3, LatGPS.3 and LonGPS.3) all contained a data gaps from 095300Z – 102600Z.  The RINU Inertial-GPS blended position is default, AltI-GPS.1 and Lat/LonI-GPS.1.

SPECIAL NOTE:    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 59 GPS dropsondes, 23 AXBT’s and 4 CTD’s deployed from the aircraft internally.