Flight ID: 20120201N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3

Attack Angle		PDAlpha.2

Dew Point Probe						TD.1

Dynamic Pressure						PQM.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE.1

Static Pressure							PSM.2

Slip Angle		PDBeta.2

Temperature Probe						TTM.1

Constants File		49cal102

Flight Directory		acdata/MET/2012/20120201N1

Local Met Data:			Takeoff (0745Z)		Landing (1445Z)

Aircraft Static Pressure		1015.1mb			1012.5mb

Tower Pressure (corrected)		1016.2mb			1014.5mb

Notes:

AltRa.1 has a dipping spike at 09:01:49z with amplitude of about 200m.

AltRa1.c has various data gaps throughout the whole flight.

TTM.3 is showing an oscillation with amplitude of 0.7 deg C.  This has been showing on flights for some time now.  TTM.1 is chosen for the reference.

TDM.1 and TDM.2 are both not rated for use under -50 deg C.  Cannot be considered reliable for dewpoints colder than -50C

PSM.1 and PSM.2 difference at takeoff = 1.2mb

PSM.1 and PSM.2 difference at landing = 1.1mb

GDIFF.2 has more inertial drift when comparing to GDIFF.1 throughout the flight.  Therefore INE.1 will be chosen as reference.

UWZ Mean is .41m/s averaged over the time at cruise altitude after processing with reference parameters.  The mean vertical winds throughout the flight should be close to zero.  Currently looking into why this value is a bit high.

After the aircraft comes out of a roll at 11:25:40z, PQBETA.2 begins to drift over the next 1min 10sec and abruptly shifts back to its normal difference from PQBETA.1.  This can be seen when comparing the difference between PQBETA.1 and PQBETA.2.

17 dropsondes launched (AVAPS II):  17 good, 0 bad