Flight ID: 20120203I1

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.1

Dew Point Probe						TDM.2X

Static Pressure							PSM.2

Dynamic Pressure						PQM.2

Altitude (for vertical wind)					AltI-GPS.1X

Flight Directory		acdata/MET/2012/20120203I1

Constants File	              20120203I1/AAMPSConfig/core/n43.xml

Local Met Data:			Takeoff (1447Z)		Landing (2140Z)

Aircraft Static Pressure (PSM.2)	1001.2 mb			1003.0 mb

Tower Pressure (corrected)		1000.8 mb			1000.6 mb

Notes:

There were no data gaps.

AltI-GPS.1 drifts slightly from the rest of the altitude sources from 145111Z – 152335Z but performs best for the remainder of the flight.  During this time AltI-GPS.1 was replaced with AltI-GPS.2 using AltI-GPS.1X = AltI-GPS.2.  AltGPS.1 and 2 both exhibit a handful of spikes throughout the flight, AltI-GPS.1X is default.

Dew point sensor 2, TdM.2 (EdgeTech) performed best, but exhibited higher spikes during the following time periods:  161745Z - 161951Z, 163611Z – 164622Z, where TdM.2X = TdM.1 + 0.  Dew point values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.

Radome variables PQM.4, PTR, PDAlpha.2, PDBeta.2 are all erroneous while the radome was iced over during the following times:  1551Z – 1625Z and 2039Z through the end of the flight.  PQM.2, PDAlpha and PDBeta # 1 are all default.

SPECIAL NOTE!!!    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds , vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 21 GPS dropsondes, and 4 AXBT’s deployed from the aircraft internally.