Flight ID: 20120209I1

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.1

Dew Point Probe						TDM.1

Static Pressure							PSM.2

Dynamic Pressure						PQM.2

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2012/20120209I1

Constants File	              20120209I1/AAMPSConfig/core/n43.xml

Local Met Data:			Takeoff (1654Z)		Landing (2243Z)

Aircraft Static Pressure (PSM.2)	997.1 mb			993.6 mb

Tower Pressure (corrected)		995.2 mb			992.5 mb

Notes:

There were no data gaps.

Radome variables PQM.4, PTR, PDAlpha.2 and PDBeta.2 are all erroneous while the radome was iced over from icing conditions from 180805 – 191427Z, and 205614Z until the end of the flight.  PQM.2, PDAlpha and PDBeta # 1 are all default.

Dewpointer #2, TDM.2 (Edgetech) was not trending well throughout several portions of the flight, and spiked from 201916Z – 203229Z, likely due to issues with the probe heater.  TDM.1, the Buck, is used as default for this flight.  Dew point values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.

There was no Novatel GPS Data, AltGPS.3, LatGPS.3 and LonGPS.3 recorded during this flight.  The RINU blended Intertial-GPS values are default.

SPECIAL NOTE!!!    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds, vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 19 GPS dropsondes, and 4 AXBT deployed from the aircraft internally.