Flight ID: 20120215N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3

Attack Angle		PDAlpha.2

Dew Point Probe						TD.1

Dynamic Pressure						PQM.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE.1

Static Pressure							PSM.2

Slip Angle		PDBeta.2

Temperature Probe						TTM.1

Constants File		49cal102

Flight Directory		acdata/MET/2012/20120215N1

Local Met Data:			Takeoff (0738Z)		Landing (1453Z)

Aircraft Static Pressure		1018.9mb			1017.8mb

Tower Pressure (corrected)		1020.3mb			1018.6mb

Notes:

Data gaps appear in Alt Ra1.c approximately every 90 seconds.  These regular gaps are not seen in AltRA.1 but there is one dropout at 08:04.47z.

When looking at TA.c and TaADDU.1 the two seem to drift apart over the cruise altitude period of the flight.  At the beginning there looks to be a difference of 1.3 deg C between the two and increases near the end of the flight to 2.3 deg C.

TTM.3 is showing an oscillation with amplitude of 0.6 deg C. TTM.1 is chosen for the reference.

TDM.1 and TDM.2 are both not rated for use under -50 deg C.  Cannot be considered reliable for dewpoints colder than -50C.

PSM.1 and PSM.2 difference at takeoff = 0.8mb

PSM.1 and PSM.2 difference at landing = 0.7mb

GDIFF.1 has less inertial drift when comparing to GDIFF.2 throughout the flight.  Therefore INE.1 will be chosen as reference.

UWZ Mean is .20m/s averaged over the time at cruise altitude after processing with reference parameters.  The mean vertical winds throughout the flight should be close to zero.

16 dropsondes launched (AVAPS II):  16 good, 0 bad