Flight ID: 20120217N1

Sensor or system						Number or Name

Accelerometer							AccZI.1

Altitude 							AltGPS.3

Attack Angle		PDAlpha.2

Dew Point Probe						TD.1

Dynamic Pressure						PQM.2

Geopotential Altitude						ALTGA.1 (AltGPS.3)

Inertial Selected						INE.1

Static Pressure							PSM.2

Slip Angle		PDBeta.2

Temperature Probe						TTM.1

Constants File		49cal102

Flight Directory		acdata/MET/2012/20120217N1

Local Met Data:			Takeoff (1927Z)		Landing (0253Z)

Aircraft Static Pressure		1022.4mb			1018.1mb

Tower Pressure (corrected)		1023.0mb			1019.9mb

Notes:

Data gaps appear in AltRa1.c approximately every 90 seconds.

When looking at TA.c and TaADDU.1 the two seem to drift apart over the cruise altitude period of the flight.  At the beginning there looks to be a difference of 1.3 deg C between the two and increases near the end of the flight to 2.4 deg C.

TTM.3 is showing an oscillation with amplitude of 0.6 deg C. TTM.1 is chosen for the reference.

TDM.1 and TDM.2 are both not rated for use under -50 deg C.  Cannot be considered reliable for dewpoints colder than -50C.

PSM.1 and PSM.2 difference at takeoff = 0.9mb

PSM.1 and PSM.2 difference at landing = 1.0mb

18 dropsondes launched (AVAPS II):  17 good, 1 bad