NOAA G-IV N49RF ERROR SUMMARY

1

WINTER STORMS 2012

2 Mar 2012 TRACK 28 (28WSC)

NCEP (PWT) PANC PANC

Flight ID: 20120302N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.1

Dew Point Probe						TdM.1

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE2

Static Pressure						PSM.2

Slip Angle							SA.1

Temperature Probe				                     TTM.1

True Airspeed						TASF.3

Constants File                                                           49cal102

Flight Directory						acdata/2012/MET/20120302N1

Local Met Data			Takeoff (1924z)		Landing (0311z)

Aircraft Static Pressure		  988.2 mb			     990.1 mb

Tower Pressure (corrected)	  987.8 mb			     990.0 mb

__________________________________________________________________

Notes:

There was a 45 second gap that began eight seconds after 00z in all AAMPS data (both analog and inertial).

In creation of higher MET parameters, PDAlphaF.1 was selected (under Attack Pressure Probe Selection choices) to utilize AA.1.

GDIFF check: Mean of 6508 meters of drift for INE #1 (with respect to the Novatel…a much larger than normal amount of drift) 1830 meters for INE#2. In creation of higher MET parameters, INE #2 selected.

TTM.3 displayed an oscillation of around 0.5C (with a period of approx 2.5 minutes) during cruise above 41K. TTM4 oscillation was very small (around 0.1C with a period of just over 10 seconds). TTM.1 is the default.

Using AA.1 by selecting PDAlphaF.1 in the calculation of higher MET parameters, Vertical Winds during the cruise portion of flight from 20:40z to 02:27z at or above 41K showed a mean UWZ of +0.11 m/s. The UWZ off the plane (A.nc using AA.2) file showed +0.19. When selecting PDAlphaF.2 using ACAT-4, the UWZ was as high as +0.33 m/s using INE2 and +0.19 using INE1.

All other flight level instruments worked optimally during the flight.

- 19 drop points assigned by NCEP

- 20 AVAPS II dropsondes deployed (Drop #11 at 2208z was bad, backup was good)

- 19 drops good and transmitted for ingestion into 03/0000z model runs

Flight Director: Richard Henning (813) 828-3310 ext. 3086 and Ian Sears