NOAA G-IV N49RF ERROR SUMMARY

1

WINTER STORMS 2012

6 Mar 2012 TRACK 28 (29WSC)

NCEP (PWT) PANC PANC

Flight ID: 20120306N1

Sensor or system						Number or Name

Accelerometer						AccZI.1

Altitude							AltGPS.3

Attack Angle						AA.1

Dew Point Probe						TdM.1

Dynamic Pressure						PQF.2

Geopotential Altitude					AltGPS.3

Inertial Selected						INE1

Static Pressure						PSM.2

Slip Angle							SA.1

Temperature Probe				                     TTM.1

True Airspeed						TASF.3

Constants File                                                           49cal102

Flight Directory						acdata/2012/MET/20120306N1

Local Met Data			Takeoff (1922z)		Landing (0317z)

Aircraft Static Pressure		  1005.5 mb			     1005.3 mb

Tower Pressure (corrected)	  1005.6 mb			     1005.3 mb

__________________________________________________________________

Notes:

There was a 45 second gap that began eight seconds after 00z in all AAMPS data (both analog and inertial).

In creation of higher MET parameters, PDAlphaF.1 was selected (under Attack Pressure Probe Selection choices) to utilize AA.1.

GDIFF check: Mean of 1364 meters of drift for INE #1 (with respect to the Novatel)

2278 meters for INE#2. In creation of higher MET parameters, INE #1 selected.

TTM.3 displayed an oscillation of around 0.5C (with a period of approx 2.5 minutes) during cruise above 41K. TTM4 oscillation was very small (around 0.1C). TTM.1 is the default.

Using AA.1 by selecting PDAlphaF.1 in the calculation of higher MET parameters, Vertical Winds during the cruise portion of flight from 20:51z to 02:38z at or above 41K showed a mean UWZ of -0.02 m/s. The UWZ off the plane (A.nc using AA.2) file showed +0.18. When selecting PDAlphaF.2 using ACAT-4, the UWZ was as high as +0.32 m/s.

All other flight level instruments worked optimally during the flight.

- 19 drop points assigned by NCEP

- 20 AVAPS II dropsondes deployed (Drop #11 at 2356z was a fast fall, backup was good)

- 19 drops good and transmitted for ingestion into 07/0000z model runs

Flight Director: Richard Henning (813) 828-3310 ext. 3086 and Ian Sears