Flight ID: 20120907H2

Sensor or system						Number or Name

Inertial Selected (for wind derivation)			INE 1

Accelerometer							AccZfilterI-GPS.1

Temperature Probe						TTM.2

Dew Point Probe						TDM.2X

Static Pressure							PSM.2

Dynamic Pressure						PQM.2

Altitude (for vertical wind)					AltI-GPS.1

Flight Directory		acdata/MET/2012/20120907H2

Constants File	              20120907H2/AAMPSConfig/core/n42.xml

Local Met Data:			Takeoff (2006Z)		Landing (0506Z)

Aircraft Static Pressure (PSM.2)	1014.9 mb			1012.8 mb

Tower Pressure (corrected)		1013.9 mb			1013.5 mb

Notes:

There was a data gap in all parameters from 00:00:07Z – 00:00:50Z.

The Edgetech dewpoint, TDM.2, performed best and was used as default.   However, it spiked erroneously twice from 00:20:09Z – 00:22:41Z and 03:52:42Z – 03:56:15Z, when TDM.1 did not.  Because of this, during these times the following substitution was made:  TDM.2X = TDM.1  Dew point values intermittently exceeded ambient temperature values during portions of flight where the aircraft was in precipitation, causing RH values greater than 100%.

The Pro-Sensing SFMR data was extremely sporadic and questionable from 00:56Z – 01:37Z due to improper auxiliary data getting to the instrument.  This is an anomaly.

SPECIAL NOTE!!!    The variable names GSZ_DPJ, ASZ_DPJ and WSZ_DPJ in the netCDF file represent vertical ground speeds, vertical air speeds and vertical wind speeds, respectively, computed using Dave Jorgensen’s vertical wind algorithm.  It is recommended that these values be used for vertical wind analysis.

All other AOC instruments worked properly.

There were 15 GPS dropsondes (14 good) and 10 AXBT’s released from the aircraft.  There were 6 hurricane penetrations.

During the last half hour of the flight, the transit back to KFLL, from 04:42:43Z until landing, several one second data gaps were observed in only some derived and reference parameters (with a .d or ref extension).